Фрагменты исходного кода программы:

CFSV.m

%> @file CFSV.m

%> @author  Vladimir Dneprov <vvdneprov@gmail.com>

%> Moscow Power Engineering Institute

%>

%> @section LICENSE

%>

%> This program is free software; you can redistribute it and/or

%> modify it under the terms of the GNU General Public License as

%> published by the Free Software Foundation; either version 2 of

%> the License, or (at your option) any later version.

%>

%> This program is distributed in the hope that it will be useful, but

%> WITHOUT ANY WARRANTY; without even the implied warranty of

%> MERCHANTABILITY or FITNESS FOR A PARTICULAR PURPOSE. See the GNU

%> General Public License for more details at

%> http://www.gnu.org/copyleft/gpl.html

%>

%> @section DESCRIPTION

%>

%> Class for Rohde&Schwarz FSV signal & spectrum analyzers\n

%> Features:\n

%> 1) Power measurement in the given bandwidth\n

%> 2) Setup center frequency of the spectrum analysis\n

%> 3) Setup bandwidth of the spectrum analysis\n

%> @brief Class for Rohde&Schwarz FSV signal & spectrum analyzers

classdef CFSV < handle

properties

%> Pointer of TCP/IP connection

Instr

%> Band for spectrum analysis

Span

%> Central frequency for spectrum analysis

CenterFreq

end

methods

%> @brief Consturctor of this class

%>

%> This is constructor of FSV control class\n

%> Example: FSV3 = CFSV;

%>

%> @return RS Object of this class

function RS = CFSV

end

%> @brief Connect to unit by local network

%>

%> Example: FSV3.SetConnection('192.168.1.100', 5025);

%> @param IP String IP-address of measurement unit

%> @param port Port for TCP/IP connection, usually 5025

%> @return Status Is 0 - fail; 1 - ok.

function Status = SetConnection(RS, IP, port)

Status = 0;

RS.Instr = tcpip(IP, port);

%Uncomment to change tcpip object's R/W buffer size.

%Default = 512 bytes

%set(RS.Instr, InputBufferSize, 2000);

%set(RS.Instr, OutputBufferSize, 2000);

fopen(RS.Instr);

if  strcmp(get(RS.Instr,'Status'),'open')

fprintf('FSV:Connection OK\n');

Status = 1;

else

fprintf('FSV:Connection Problem');

end

end

%> @brief Close connection and return to manual control

%>

%> Example: FSV3.CloseConnection;

%> @return Status Returns a status of 0 when the close operation is successful. Otherwise, it returns -1

function Status = CloseConnection(RS)

fprintf(RS.Instr,'&GTL');

Status = fclose(RS.Instr);

end

%> @brief Send SCPI command to FSV

%>

%> Example: FSV3.SendCommand('*RST');

%> @param strCommand String of SCPI command

%> @return Status Returns a status of 1 when the operation is successful. Otherwise, it returns 0

function [Status] = SendCommand(RS, strCommand)

Status = 0;

% Check number of input args

if( nargin ~= 2 )

disp( '*** Wrong number of input arguments' )

return;

end

% Check first parameter

if( isobject(RS) ~= 1 )

disp( '*** The first parameter is not an object.' );

return;

end

% Check second parameter

if( isempty(strCommand) || (ischar(strCommand)~= 1) )

disp ('*** Command string is empty or not a string.');

return;

end

% Command sending

fprintf (RS.Instr, strCommand);

Error = QueryError(RS);

if (Error==1)

return;

end

Status = 1;

end

%> @brief Send request for answer

%>

%> Example: FSV3.SendQuery('*IDN?');

%> @param strCommand String of SCPI command

%> @return Status Returns a status of 1 when the operation is successful. Otherwise, it returns 0

%> @return Result Returns an answer of FSV

function [Status, Result] = SendQuery(RS, strCommand)

Status = 0;

Result = '';

% Check number of input args

if( nargin ~= 2 )

disp( '*** Wrong number of input arguments' )

return;

end

% Check first parameter

if( isobject(RS) ~= 1 )

disp( '*** The first parameter is not an object.' );

return;

end

% Check second parameter

if( isempty(strCommand) || (ischar(strCommand)~= 1) )

disp ('*** Command string is empty or not a string.');

return;

end

% Check: is strCommand the question?

if( isempty( strfind(strCommand, '?' ) ) )

disp( '*** Queries must end with a question mark.' );

return;

end

% Send request and receive answer

fprintf (RS.Instr, strCommand);

Result = fscanf(RS.Instr, '%c');

Status = 1;

end

%> @brief Get information about instrument

%>

%> Example: FSV3.GetIDN;

%> @return IDN String information about instrument

%> @return Status Returns a status of 1 when the operation is successful. Otherwise, it returns 0

function [Status, IDN] = GetIDN(RS)

Status = 0;

IDN = '';

[Stat, IDN] = RS.SendQuery('*IDN?');

if (Stat == 0)

return;

end

Status = 1;

end

%> @brief Preset instrument and clear errors log

%>

%> Example: FSV3.Preset;

%> @return Status Returns a status of 1 when the operation is successful. Otherwise, it returns 0

function [Status] = Preset(RS)

Status = 0;

[Stat] = RS.SendCommand('*RST; *CLS');

if (Stat == 0)

return;

end

[status, result] = RS.SendQuery('*OPC?');

if (status == 0 || result(1)~='1')

return;

end

Status = 1;

end

%> @brief Check errors in FSV

%>

%> Example: FSV3.QueryError;

%> @return Err Err = 1 instrument error occured, 0 no error

function [Err] = QueryError(RS)

Result = '1';

Counter = 0;

Err = 0;

% query for errors in a loop until "0, No error" is returned

% and limit the number of iterations to 100

while (Result(1) ~= '0' && Counter < 100)

[status, Result] = RS.SendQuery('SYST:ERR?');

if (status == 0)

disp( '*** Error occurred' );

Err = 1;

end

if (Result(1)~='0')

disp (['*** Instrument Error: ' Result]);

Err = 1;

end

Counter = Counter + 1;

end

end

%> @brief Set the center frequency of the spectrum analysis

%>

%> Example:\n FSV3.SetCenterFreq('1GHz');\n

%> FSV3.SetCenterFreq(1E9);

%> @param Freq Central frequency string 1GHz or float 1.11E9 (Hz)

%> @return Status Returns a status of 1 when the operation is successful. Otherwise, it returns 0

function [Status] = SetCenterFreq(RS, Freq)

Status = 0;

if (ischar(Freq))

[Stat] = RS.SendCommand(['FREQ:CENT ' Freq]);

if ( Stat == 0)

return;

end

else

[Stat] = RS.SendCommand(sprintf('FREQ:CENT %.5f',Freq));

if ( Stat == 0)

return;

end

end

RS.CenterFreq = Freq;

Status = 1;

end

%> @brief Set bandwidth of the spectrum analysis

%>

%> Example:\n FSV3.SetSpan('1MHz');\n

%> FSV3.SetSpan(1E6);

%> @param Span String 1MHz or float 1.1E6

%> @return Status Returns a status of 1 when the operation is successful. Otherwise, it returns 0

function [Status] = SetSpan(RS, Span)

Status = 0;

if (ischar(Span))

[Stat] = RS.SendCommand(['FREQ:SPAN ' Span]);

if ( Stat == 0)

return;

end

else

[Stat] = RS.SendCommand(sprintf('FREQ:SPAN %.5f',Span));

if ( Stat == 0)

return;

end

end

RS.Span = Span;

Status = 1;

end

%> @brief Measure power in given bandwith

%>

%> Example:\n FSV3.PowerMeasure('10MHz');\n

%> FSV3.PowerMeasure(1E7);

%> @param Bandwidth String 1MHz or float 1.123E6 (Hz)

%> @return measure Power in bandwith, dBm

%> @return Status Returns a status of 1 when the operation is successful. Otherwise, it returns 0

function [Status, measure] = PowerMeasure(RS, Bandwidth)

Status = 0;

if (ischar(Bandwidth))

[Stat] = RS.SendCommand(['SENS:POW:ACH:BAND:CHAN1 ', Bandwidth]);

if ( Stat == 0)

return;

end

else

[Stat] = RS.SendCommand(sprintf('SENS:POW:ACH:BAND:CHAN1 %.5f', Bandwidth));

if ( Stat == 0)

return;

end

end

[status, result] = RS.SendQuery('*OPC?');

if (status == 0 || result(1)~='1'),

return;

end

[Stat] = RS.SendCommand('INIT:CONT OFF');

if ( Stat == 0)

return;

end

[Stat] = RS.SendCommand('INIT;*WAI');

if ( Stat == 0)

return;

end

[m, measure] = RS.SendQuery('CALC:MARK:FUNC:POW:RES? CPOW');

if (m == 0 || result(1)~='1'),

return;

end

Status = 1;

disp(['Measure result:' measure]);

end

end

end

CReceiver.m

%> @file CReceiver.m

%> @author  Vladimir Dneprov <vvdneprov@gmail.com>

%> Moscow Power Engineering Institute

%>

%> @section LICENSE

%>

%> This program is free software; you can redistribute it and/or

%> modify it under the terms of the GNU General Public License as

%> published by the Free Software Foundation; either version 2 of

%> the License, or (at your option) any later version.

%>

%> This program is distributed in the hope that it will be useful, but

%> WITHOUT ANY WARRANTY; without even the implied warranty of

%> MERCHANTABILITY or FITNESS FOR A PARTICULAR PURPOSE. See the GNU

%> General Public License for more details at

%> http://www.gnu.org/copyleft/gpl.html

%>

%> @section DESCRIPTION

%>

%> Class to work with navigation receiver via serial port\n

%> Features:\n

%> 1) Reset receiver ( only for GEOS - 3 )\n

%> 2) Get solution status ( only for GEOS - 3 )\n

%> 3) Read data from receiver

%> @brief Class to work with navigation receiver via serial port

classdef CReceiver < handle

properties

%> Pointer of serial connection

obj

%> Type of solution (1 - No solution, 2 - 2D, 3 - 3D)

FixType

end

methods

%> @brief Consturctor of this class

%>

%> This is constructor of Receiver control class\n

%> Example: Rec1 = CReceiver;

%>

%> @return RCV Object of this class

function RCV = CReceiver

end

%> @brief Set parameters of serial port

%>

%> Example: Rec1.SerialConfig('COM7', 115200);

%> @param COM String port name

%> @param Baud Baud rate

function SerialConfig(RCV, COM, Baud)

if ( nargin ~= 3)

disp('***Wrong number of input arguments')

return;

end

if ( isempty(COM) || (ischar(COM)~= 1) )

disp('***Enter COM<x> as a string');

return;

end

if (isempty(Baud))

disp('***Enter Baud rate');

return;

end

RCV.obj = serial(COM);

set(RCV.obj,'BaudRate',Baud);

end

%> @brief Connect to receiver by serial port

%>

%> Example: Rec1.SerialConnect;

%> @return Status Is 0 - fail; 1 - ok.

function [Status] = SerialConnect(RCV)

Status = 0;

fopen(RCV.obj);

if strcmp(get(RCV.obj,'Status'),'open')

disp('***Serial: connection OK');

Status = 1;

else

disp('***Serial: connection error');

return;

end

end

%> @brief Read data from receiver

%>

%> Example: Rec1.RecieveString;

%> @return Answer Data from receiver

function [Answer] = RecieveString(RCV)

Answer = fscanf(RCV.obj);

end

%> @brief Close connection to receiver

%>

%> Example: Rec1.SerialClose;

%> @return Status Is 0 - fail; 1 - ok.

function [Status] = SerialClose(RCV)

Status = 0;

fclose(RCV.obj);

if strcpm(get(RCV.obj,'Status'),'close')

disp('***Serial: close OK');

Status = 1;

else

disp('***Serial: close error');

return;

end

end

%> @brief Reset receiver ( NMEA string is true for GEOS-3 )

%>

%> Example: Rec1.Reset;

function Reset(RCV)

fprintf(RCV.obj,'$GPSGG,CSTART*6B\n\r');

end

%> @brief Get solution status and store it in FixType

%>

%> Example: Rec1.GetSolutionStatus;

function GetSolutionStatus(RCV)

while(1)

answer = RecieveString(RCV);

if numel(answer) < 10

return;

end

if strcmp(answer(1:6),'$GNGSA')||strcmp(answer(1:6),'$GPGSA')

sol = answer(10);

switch sol

case '1'

RCV.FixType = 1;

disp('No solution')

return;

case '2'

RCV.FixType = 2;

disp('2D Fix')

return;

case '3'

RCV.FixType = 3;

disp('3D Fix')

return;

end

end

end

end

end

end

CSMBV.m

%> @file CSMBV.m

%> @author  Vladimir Dneprov <vvdneprov@gmail.com>

%> Moscow Power Engineering Institute

%>

%> @section LICENSE

%>

%> This program is free software; you can redistribute it and/or

%> modify it under the terms of the GNU General Public License as

%> published by the Free Software Foundation; either version 2 of

%> the License, or (at your option) any later version.

%>

%> This program is distributed in the hope that it will be useful, but

%> WITHOUT ANY WARRANTY; without even the implied warranty of

%> MERCHANTABILITY or FITNESS FOR A PARTICULAR PURPOSE. See the GNU

%> General Public License for more details at

%> http://www.gnu.org/copyleft/gpl.html

%>

%> @section DESCRIPTION

%>

%> Class for Rohde&Schwarz SMBV100A vector signal generator\n

%> Features:\n

%> 1) Set up output power\n

%> 2) Set up carrier frequency\n

%> 3) Simulate GPS signals of the given number of satellites\n

%> @brief Class for Rohde&Schwarz SMBV100A vector signal generator

classdef CSMBV < handle

properties

%> Frequency of the carrier Hz

Freq

%> Output power dBm

Level

%> Pointer of TCP/IP connection

Instr

end

methods

%> @brief Consturctor of this class

%>

%> This is constructor of SMBV control class\n

%> Example: SMBV1 = CSMBV;

%>

%> @return RS Object of this class

function RS = CSMBV

end

%> @brief Connect to unit by local network

%>

%> Example: SMBV1.SetConnection('192.168.1.55', 5025);

%> @param IP String IP-address of unit

%> @param port Port for TCP/IP connection, usually 5025

%> @return Status Is 0 - fail; 1 - ok.

function Status = SetConnection(RS, IP, port)

Status = 0;

RS.Instr = tcpip(IP, port);

%Uncomment to change tcpip object's R/W buffer size.

%Default = 512 bytes

%set(RS.Instr, InputBufferSize, 2000);

%set(RS.Instr, OutputBufferSize, 2000);

fopen(RS.Instr);

if  strcmp(get(RS.Instr,'Status'),'open')

fprintf('SMBV:Connection OK\n');

Status = 1;

else

fprintf('SMBV:Connection Problem');

end

end

%> @brief Close connection and return to manual control

%>

%> Example: SMBV1.CloseConnection;

%> @return Status Returns a status of 0 when the close operation is successful. Otherwise, it returns -1

function Status = CloseConnection(RS)

fprintf(RS.Instr,'&GTL');

Status = fclose(RS.Instr);

end

%> @brief Send SCPI command to SMBV

%>

%> Example: SMBV1.SendCommand('*RST');

%> @param strCommand String of SCPI command

%> @return Status Returns a status of 1 when the operation is successful. Otherwise, it returns 0

function [Status] = SendCommand(RS, strCommand)

Status = 0;

% Check number of input args

if( nargin ~= 2 )

disp( '*** Wrong number of input arguments' )

return;

end

% Check first parameter

if( isobject(RS) ~= 1 )

disp( '*** The first parameter is not an object.' );

return;

end

% Check second parameter

if( isempty(strCommand) || (ischar(strCommand)~= 1) )

disp ('*** Command string is empty or not a string.');

return;

end

% Command sending

fprintf (RS.Instr, strCommand);

Error = QueryError(RS);

if (Error==1)

return;

end

Status = 1;

end

%> @brief Send request for answer

%>

%> Example: SMBV1.SendQuery('*OPC?');

%> @param strCommand String of SCPI command

%> @return Status Returns a status of 1 when the operation is successful. Otherwise, it returns 0

%> @return Result Returns a answer of SMBV

function [Status, Result] = SendQuery(RS, strCommand)

Status = 0;

Result = '';

% Check number of input args

if( nargin ~= 2 )

disp( '*** Wrong number of input arguments' )

return;

end

% Check first parameter

if( isobject(RS) ~= 1 )

disp( '*** The first parameter is not an object.' );

return;

end

% Check second parameter

if( isempty(strCommand) || (ischar(strCommand)~= 1) )

disp ('*** Command string is empty or not a string.');

return;

end

% Check: is strCommand the question?

if( isempty( strfind(strCommand, '?' ) ) )

disp( '*** Queries must end with a question mark.' );

return;

end

% Send request and receive answer

fprintf (RS.Instr, strCommand);

Result = fscanf(RS.Instr, '%c');

Status = 1;

end

%> @brief Get information about instrument

%>

%> Example: SMBV1.GetIDN;

%> @return IDN String information about instrument

%> @return Status Returns a status of 1 when the operation is successful. Otherwise, it returns 0

function [Status, IDN] = GetIDN(RS)

Status = 0;

IDN = '';

[Stat, IDN] = RS.SendQuery('*IDN?');

if (Stat == 0)

return;

end

Status = 1;

end

%> @brief Preset instrument and clear errors log

%>

%> Example: SMBV1.Preset;

%> @return Status Returns a status of 1 when the operation is successful. Otherwise, it returns 0

function [Status] = Preset(RS)

Status = 0;

[Stat] = RS.SendCommand('*RST; *CLS');

if (Stat == 0)

return;

end

[status, result] = RS.SendQuery('*OPC?');

if (status == 0 || result(1)~='1')

return;

end

Status = 1;

end

%> @brief Check errors in SMBV

%>

%> Example: SMBV1.QueryError;

%> @return Err Err = 1 instrument error occured, 0 no error

function [Err] = QueryError(RS)

Result = '1';

Counter = 0;

Err = 0;

% query for errors in a loop until "0, No error" is returned

% and limit the number of iterations to 100

while (Result(1) ~= '0' && Counter < 100)

[status, Result] = RS.SendQuery('SYST:ERR?');

if (status == 0)

disp( '*** Error occurred' );

Err = 1;

break;

end

if (Result(1)~='0')

disp (['*** Instrument Error: ' Result]);

Err = 1;

end

Counter = Counter + 1;

end

end

%> @brief Set the output power level

%>

%> Example: SMBV1.SetLevel(-130);

%> @param Level Output power level, dBm

%> @return Status Returns a status of 1 when the operation is successful. Otherwise, it returns 0

function [Status] = SetLevel(RS, Level)

Status = 0;

RS.Level = Level;

[Stat] = RS.SendCommand(sprintf('SOUR:POW %.1f', Level));

if (Stat == 0)

return;

end

Status = 1;

end

%> @brief Set the frequency of the carrier

%>

%> Example:\n SMBV1.SetFreq('1GHz');\n

%> SMBV1.SetFreq('1E9');\n

%> @param Freq Frequency of the carrier. Freq can be String 10GHz or float(Hz) 10.12345E9

%> @return Status Returns a status of 1 when the operation is successful. Otherwise, it returns 0

function [Status] = SetFreq(RS, Freq)

Status = 0;

if (ischar(Freq))

[Stat] = RS.SendCommand(['SOUR:FREQ ' Freq]);

RS.Freq = str2num(Freq);

if ( Stat == 0)

return;

end

else

[Stat] = RS.SendCommand(sprintf('SOUR:FREQ %.5f', Freq));

RS.Freq = Freq;

if (Stat == 0)

return;

end

end

RS.CenterFreq = Freq;

Status = 1;

end

%> @brief Set the RF output ON/OFF

%>

%> Example: SMBV1.SetRFOutput('ON');

%> @param State String ON/OFF

%> @return Status Returns a status of 1 when the operation is successful. Otherwise, it returns 0

function [Status] = SetRFOutput(RS, State)

Status = 0;

if (ischar(State))

[Stat] = RS.SendCommand(['OUTP ' State]);

if (Stat == 0)

return;

end

else

disp('Output state is string ON/OFF');

return;

end

Status = 1;

end

%> @brief Simulate signals of several GPS satellites, satellite signal power is the same, the frequency range is L1, location - Moscow

%>

%> Example: SMBV1.SetGPS(6);

%> @param SatNumber Integer number of satellites

%> @return Status Returns a status of 1 when the operation is successful. Otherwise, it returns 0

function [Status] = SetGPS(RS, SatNumber)

Status = 0;

%           RS.RefPoW = RefPow;

[Stat] = RS.SendCommand('SOUR:BB:GPS:PRES');

if (Stat == 0)

return;

end

[Stat, result] = RS.SendQuery('*OPC?');

if (Stat == 0 || result(1)~='1')

return;

end

[Stat] = RS.SendCommand('SOUR:BB:GPS:STAT OFF');

if (Stat == 0)

return;

end

[Stat] = RS.SendCommand('SOUR:BB:GPS:SMODE USER');

if (Stat == 0)

return;

end

[Stat] = RS.SendCommand('SOUR:BB:GPS:LOC:SEL "Moscow"');

if (Stat == 0)

return;

end

%           [status] = RS.sendCommand(sprintf('SOUR:BB:GPS:POWER:REF %.1f', RefPow));

%           if (status<0)

%               return;

%           end

[Stat] = RS.SendCommand(sprintf('BB:GPS:SAT:COUN %i', SatNumber));

if (Stat == 0)

return;

end

[Stat] = RS.SendCommand('SOUR:BB:GPS:GOC');

if (Stat == 0)

return;

end

[Stat, result] = RS.SendQuery('*OPC?');

if (Stat == 0 || result(1)~='1')

return;

end

[Error] = RS.QueryError;

if (Error == 1)

return;

end

[Stat] = RS.SendCommand('SOUR:BB:GPS:STAT ON');

if (Stat == 0)

return;

end

progress = '0';

[Stat, progress] = RS.SendQuery('SOUR:BB:PROG:MCOD?');

if (Stat == 0)

return;

end

while (str2num(progress) ~= 100)

[Stat, progress] = RS.SendQuery('SOUR:BB:PROG:MCOD?');

if (Stat == 0)

return;

end

end

[Stat, result] = RS.SendQuery('*OPC?');

if (Stat == 0 || result(1)~='1')

return;

end

[Stat] = RS.SendCommand('BB:GPS:TRIGger:EXECute');

if (Stat == 0)

return;

end

[Error] = RS.QueryError;

if (Error == 1)

return;

end

Status = 1;

end

end

end

CRSC.m

%> @file CRSC.m

%> @author  Vladimir Dneprov <vvdneprov@gmail.com>

%> Moscow Power Engineering Institute

%>

%> @section LICENSE

%>

%> This program is free software; you can redistribute it and/or

%> modify it under the terms of the GNU General Public License as

%> published by the Free Software Foundation; either version 2 of

%> the License, or (at your option) any later version.

%>

%> This program is distributed in the hope that it will be useful, but

%> WITHOUT ANY WARRANTY; without even the implied warranty of

%> MERCHANTABILITY or FITNESS FOR A PARTICULAR PURPOSE. See the GNU

%> General Public License for more details at

%> http://www.gnu.org/copyleft/gpl.html

%>

%> @section DESCRIPTION

%>

%> Class for Rohde&Schwarz RSC attenuator\n

%> Features:\n

%> 1) Set up attenuation

%> @brief Class for Rohde&Schwarz RSC attenuator

classdef CRSC < handle

properties

%> Pointer of TCP/IP connection

Instr

%> Attenuation dB

Attenuation

end

methods

%> @brief Consturctor of this class

%>

%> This is constructor of RSC control class\n

%> Example: RSC3 = CRSC;

%>

%> @return RS Object of this class

function RS = CRSC

end

%> @brief Connect to unit by local network

%>

%> Example: RSC3.SetConnection('192.168.1.58', 5025);

%> @param IP String IP-address of measurement unit

%> @param port Port for TCP/IP connection, usually 5025

%> @return Status Is 0 - fail; 1 - ok.

function Status = SetConnection(RS, IP, port)

Status = 0;

RS.Instr = tcpip(IP, port);

%Uncomment to change tcpip object's R/W buffer size.

%Default = 512 bytes

%set(RS.Instr, InputBufferSize, 2000);

%set(RS.Instr, OutputBufferSize, 2000);

fopen(RS.Instr);

if  strcmp(get(RS.Instr,'Status'),'open')

fprintf('RSC:Connection OK\n');

Status = 1;

else

fprintf('RSC:Connection Problem');

end

end

%> @brief Close connection and return to manual control

%>

%> Example: RSC3.CloseConnection;

%> @return Status Returns a status of 0 when the close operation is successful. Otherwise, it returns -1

function Status = CloseConnection(RS)

fprintf(RS.Instr,'&GTL');

Status = fclose(RS.Instr);

end

%> @brief Send SCPI command to RSC

%>

%> Example: RSC3.SendCommand('*RST');

%> @param strCommand String of SCPI command

%> @return Status Returns a status of 1 when the operation is successful. Otherwise, it returns 0

function [Status] = SendCommand(RS, strCommand)

Status = 0;

% Check number of input args

if( nargin ~= 2 )

disp( '*** Wrong number of input arguments' )

return;

end

% Check first parameter

if( isobject(RS) ~= 1 )

disp( '*** The first parameter is not an object.' );

return;

end

% Check second parameter

if( isempty(strCommand) || (ischar(strCommand)~= 1) )

disp ('*** Command string is empty or not a string.');

return;

end

% Command sending

fprintf (RS.Instr, strCommand);

Error = QueryError(RS);

if (Error==1)

return;

end

Status = 1;

end

%> @brief Send request for answer

%>

%> Example: RSC3.SendQuery('*IDN?');

%> @param strCommand String of SCPI command

%> @return Status Returns a status of 1 when the operation is successful. Otherwise, it returns 0

%> @return Result Returns an answer of FSV

function [Status, Result] = SendQuery(RS, strCommand)

Status = 0;

Result = '';

% Check number of input args

if( nargin ~= 2 )

disp( '*** Wrong number of input arguments' )

return;

end

% Check first parameter

if( isobject(RS) ~= 1 )

disp( '*** The first parameter is not an object.' );

return;

end

% Check second parameter

if( isempty(strCommand) || (ischar(strCommand)~= 1) )

disp ('*** Command string is empty or not a string.');

return;

end

% Check: is strCommand the question?

if( isempty( strfind(strCommand, '?' ) ) )

disp( '*** Queries must end with a question mark.' );

return;

end

% Send request and receive answer

fprintf (RS.Instr, strCommand);

Result = fscanf(RS.Instr, '%c');

Status = 1;

end

%> @brief Get information about instrument

%>

%> Example: RSC3.GetIDN;

%> @return IDN String information about instrument

%> @return Status Returns a status of 1 when the operation is successful. Otherwise, it returns 0

function [Status, IDN] = GetIDN(RS)

Status = 0;

IDN = '';

[Stat, IDN] = RS.SendQuery('*IDN?');

if (Stat == 0)

return;

end

Status = 1;

end

%> @brief Preset instrument and clear errors log

%>

%> Example: RSC3.Preset;

%> @return Status Returns a status of 1 when the operation is successful. Otherwise, it returns 0

function [Status] = Preset(RS)

Status = 0;

[Stat] = RS.SendCommand('*RST; *CLS');

if (Stat == 0)

return;

end

[status, result] = RS.SendQuery('*OPC?');

if (status == 0 || result(1)~='1')

return;

end

Status = 1;

end

%> @brief Check errors in RSC

%>

%> Example: RSC3.QueryError;

%> @return Err Err = 1 instrument error occured, 0 no error

function [Err] = QueryError(RS)

Result = '1';

Counter = 0;

Err = 0;

% query for errors in a loop until "0, No error" is returned

% and limit the number of iterations to 100

while (Result(1) ~= '0' && Counter < 100)

[status, Result] = RS.SendQuery('SYST:ERR?');

if (status == 0)

disp( '*** Error occurred' );

Err = 1;

end

if (Result(1)~='0')

disp (['*** Instrument Error: ' Result]);

Err = 1;

end

Counter = Counter + 1;

end

end

%> @brief Set the attenuation

%>

%> Example: RSC3.SetAttenuation(30);

%> @param ATT attenuation dB

%> @return Status Returns a status of 1 when the operation is successful. Otherwise, it returns 0

function [Status] = SetAttenuation(RS, ATT)

Status = 0;

[Stat] = RS.SendCommand(sprintf('ATT1:ATT %.2f', ATT));

if ( Stat == 0)

return;

end

RS.Attenuation = ATT;

Status = 1;

end

end

end

Sensivity.m

SMBV = CSMBV;

Rec = CReceiver;

%соединение с SMBV

[Stat] = SMBV.SetConnection('192.168.1.22',5025);

if (Stat == 0)

error('Connection problem')

end

%сброс настроек SMBV в дефолтные, очистка лога ошибок

[Stat] = SMBV.Preset;

if (Stat == 0)

error('Error')

end

%запрос модели, серийного номера

[Stat, result] = SMBV.GetIDN;

if (Stat == 0)

error('Error')

end

disp(result);

%проверка системных ошибок

[status, result] = SMBV.SendQuery('SYST:SERR?');

if (result(1) ~= '0' || status == 0 )

disp (['*** Instrument error : ' result]);

return;

end

%начальные настройки эксперимента

StartLevel = -95; % стартовая мощность сигнала

%запуск имитации сигнала GPS

[Stat] = SMBV.SetGPS(6);

if (Stat == 0)

error('Error')

end

%установка стартовой мощности

[Stat] = SMBV.SetLevel(StartLevel);

if (Stat == 0)

error('Error')

end

[Stat] = SMBV.SetRFOutput('ON');

if (Stat == 0)

error('Error')

end

%Настройка соединения с приемником

Rec.SerialConfig('COM6',115200);

%Соединение с приемником

Stat = Rec.SerialConnect;

if (Stat == 0)

error('Serial: connection problem')

end

%Перезагрузка приемника, запуск отсчета времени на даннй мощности

Rec.Reset;

pause(70);

tin_thislevel = tic;

%Параметры эксперимента: шаг изменения мощности, ожидание на мощности

LevelStep = 1; PauseOnLevel = 30;

HaveFix = 0;

k = 1;

RecIsDead5sec = 0;

RecOkOnLastStep = 0;

Pow_arr = cell(1,1);

p = 1;

m = 0;

%цикл эксперимента

while (1)

Rec.GetSolutionStatus;

if (Rec.FixType == 3)

RecOkOnLastStep = 1;

HaveFix = 1;

if (toc(tin_thislevel) > PauseOnLevel)

LastOkLevel = SMBV.Level;

Pow_arr{p,1} = [LastOkLevel 1];

p = p + 1;

if (LastOkLevel <= -95 && LastOkLevel >= -118)

LevelStep = 8;

elseif (LastOkLevel == -119)

LevelStep = 2;

elseif (LastOkLevel <= -120 && LastOkLevel >= -135)

LevelStep = 0.5;

end

Stat = SMBV.SetLevel(LastOkLevel - LevelStep);

if (Stat == 0)

error('SMBV error')

end

tin_thislevel = tic;

end

elseif ((Rec.FixType == 1 || Rec.FixType == 2) && RecOkOnLastStep == 1 )

DeathTime = tic;

end

if ((Rec.FixType == 1 || Rec.FixType ==2) && HaveFix == 1 && RecOkOnLastStep == 0 )

if ( toc(DeathTime) > 5 )

RecIsDead5sec = 1;

else

RecIsDead5sec = 0;

end

end

if (Rec.FixType == 1 || Rec.FixType == 2)

RecOkOnLastStep = 0;

end

if (RecIsDead5sec == 1)

ResultLevel(k) = LastOkLevel;

k = k + 1;

Pow_arr{p,1} = [(LastOkLevel - LevelStep) 0];

p = p + 1;

file = [num2str(m) 'MSHUpower.mat'];

save(file, 'Pow_arr');

m = m + 1;

Rec.Reset;

SMBV.SetLevel(StartLevel);

HaveFix = 0;

RecOkOnLastStep = 0;

RecIsDead5sec = 0;

toc(DeathTime);

pause(70);

tin_thislevel  = tic;

end

end